10.	REACTION EQUATIONS

Clicking the Reaction Equations button in the main menu shows the Reaction Equations Window, see Fig. 1. With this calculation option you can calculate the heat capacity, enthalpy, entropy and Gibbs energy values of a single species as well as of specified reactions between pure substances.

See the reference state definitions, valid notations and abbreviations for the description of the chemical formulae in Chapter 28. Databases, chapter 2.

10.1	One Chemical Substance

Fig. 1. Reaction Equations Window of HSC Chemistry.

Fig. 2. Thermodynamic data of A12O3 (alumina) displayed by the Reaction Equation option of HSC Chemistry.

By entering a single chemical formula into the Formula box you will get similar tables of thermochemical data as presented in many thermochemical data books. HSC will, however, provide the results faster and exactly at those temperatures which you really want. Please follow these steps:

1.	Write a chemical formula into the box, Fig. 1.

For example: Fe, Na2SO4, Al2O3, SO4(-a), H(+a) or SO2(g).

See the valid notation and syntax of chemical formulae in Chapter 21.2.

2.	Select the lower limit, upper limit and temperature step.

3.	Select the Temperature and Energy Units, by clicking the corresponding buttons.

4.	Select the Format of the results.

Normal (Absolute scale):

H(species), S(species) and C(species)

This format is used for example in the famous I. Barin, O. Knacke, and O. Kubaschewski data compilation1.

Delta (Formation functions):

H	= H(species) - H(elements)

S	= S(species) - S(elements)

G	= G(species) - G(elements)

G	= G(ions) - G(elements) + z/2*G(H2(g)) - z*G(H(+a))

z	= charge.

This format is used for example in the NBS TablesNBS 82.

5.	The Collect to Sheet option will collect several tables on the same spreadsheet.

6.	Select the Show Transitions option if you also want to see the data at the phase transformation temperatures, such as crystal structure changes and melting.

7.	Select the Criss-Cobble option if you want a Criss-Cobble extrapolation for the heat capacity of aqueous species, see Chapter 21.4.

8.	Press Calculate (or Enter) to get the results on the screen.

9.	Press Print to print the results, see Fig. 2. Note that you can collect several sets of results in the same sheet if you have selected the Collect to Sheet option in Fig. 1. You can clear the whole sheet by pressing Clear.

10.	Press Copy All to get all the results into the Clipboard, then you can easily paste the results to other Windows applications, for example, to MS-Excel, see Fig. 2. Using Copy it is possible to copy and paste contents of individual cells to other applications.

11.	If you want to save the formula and results in an ASCII-file press Save, see Fig. 2. You can read these files back to the Reaction module using File Open, see Fig. 1. Note that Save saves all the selections in Fig. 1, so you can return these using File Open. The File Open HSC 2 File button reads only old HSC 2.0 files which return only formula, but not the selections nor temperature range.

Note:	1.	You can easily check the basic data from the database, which has been used in the reaction module calculations. In Fig. 1 select the formula in the Reaction Equation box and press Peep Database. The same procedure can be found in Fig. 2 by pressing the right mouse button or selecting Edit from the menu.

2.	The table in Fig. 2 has some formatting and Copy - Paste functions as do other tables in HSC Chemistry. These features help to create a good printed copy of the results for various purposes.

3.	HSC searches for the species data first from the Own database (OwnDB5.HSC). If it does not find a species there, it will search from the Main database (MainDB5.HSC). Therefore HSC always uses data in the Own database if the same species exists in both Own and Main databases.

4.	If you have selected Delta-format for the results, HSC will also search for data for the necessary elements and calculate the formation functions of enthalpy, entropy and Gibbs energy. Usually the original experimental data is in this format: however, sometimes the comparison of data in this format may be more difficult because the data sources often use different data for elements.

5.	HSC will make a Criss-Cobble extrapolation for the heat capacity of aqueous species at elevated temperatures (> 25 C°) if the Criss-Cobble option is selected. The extrapolation is not done if A and B of the heat capacity coefficients A, B, C and D exist in the HSC Chemistry databases. The extrapolation error increases rapidly at higher temperatures. More information on extrapolation is given in Chapter 28.4.

6.	For aqueous species it is recommended to set:

Lower temperature		= 25 °C

Upper temperature		= 300 °C

Step				= 25 °C

10.2	One Chemical Substance Results

After pressing the Calculate button in the previous screen, Fig. 1, you will arrive in the results window, Fig. 2. From this results screen you can save and print the results:

- Press Save if you wish to recalculate the results later. The Save button will also save the settings used in Fig. 1. You can read these files back to HSC using the File Open button, see Fig. 1.

- Press Print if you want a paper copy. If you have selected the Collect to Sheet option in Fig. 1, then all results will be collected on the same sheet and can be printed together. Clear will clear the results sheet.

- Press Copy to get the results of selected cells into the Clipboard, then you can easily paste the results, for example, to MS -Excel, see Fig. 2.

- Press Copy All to get all the results of the sheet into the Clipboard, then you can easily paste the results, for example, to MS -Excel, see Fig. 2.

- Press OK to return to the previous Window.

10.3	Reaction Equations

Fig. 3. Input data for Reaction Equation calculations.

Fig. 4. Results of Reaction Equation calculations.

You can write almost any kind of reaction equation into the Reaction Equation box of HSC, Fig. 3. Here are some examples of valid equation syntax:

2Cu + S = Cu2S

H2O = H(+a) + OH(-a)

H2(g) = 2H(+a) + 2e-

H2O = 1/2O2(g) + 2H(+a) + 2e-

Ag = Ag(+a) + e-

3NO2(-a) + 2H(+a) = 2NO(g) + H2O + NO3(-a)

2Al(+3a) + 3S(-2a) + 6H2O = 2Al(OH)3 + 3H2S(g)

Write the reaction equation into the box, see Fig. 3. If you have not given the stoichiometric coefficients for the species, you can press Balance Equation for solving unknown coefficients. The balance button solves the coefficients on the basis of element balance equations. Therefore it cannot solve unknown coefficients if their number is larger than the number of elements in the corresponding reaction.

On the right side of the button you may give a multiplier, which will be used to multiply all the coefficients in the reaction equation. The default value is 1, which means that the smallest stoichiometric coefficient in the reaction equation is 1.

You can continue in the same way as in the One Chemical Formula option Chapter 10.1. Note that the Delta Format and Show Transitions options have no effect on the results, because the enthalpy and Gibbs energy of a reaction are in the Delta format by definition.

HSC calculates the stoichiometry of the reaction given by the user, and points out errors if the element balance is incorrect.

The example in Fig. 3 refers to the Mond-process for refining impure nickel. In the process raw impure nickel is first treated with CO gas at 60 °C to evaporate the nickel as a carbonyl gas. In the second stage, the temperature of the gas is increased to 200 °C to decompose the nickel carbonyl gas into pure metallic nickel and CO. This process works because the equilibrium of this reaction is on the right side (Equilibrium constant K > 1) at lower temperatures and on the left side (K < 1) at higher temperatures. The reaction is exothermic (DH is negative) at all temperatures.

Vapor pressures p can be calculated by writing the reaction equation for the vaporization reaction concerned. For example, for pure magnesium the equilibrium is Mg = Mg(g), Fig. 5. The activity a of pure magnesium is 1 and thus the vapor pressure in bar is equal to the equilibrium constant according to Eq. (10) in Chapter  8. Introduction and Eq. (1). See also Fig. 6.

K  =  pMg / aMg  =  pMg			[1]

If a substance vaporizes into several polymers, all of them must be taken into account. The total vapor pressure is then the sum of all the individual partial pressures, if the gas phase behaves ideally.

You can also calculate more complicated reactions. First write the reaction as shown in Fig. 7, then press Balance for the coefficients, see Fig. 8 and finally press Calculate for the results, see Fig. 9. Note that for aqueous ionic reactions HSC also calculates the electrode potential versus Standard Hydrogen Electrode if electron (e-) is used in the formula.

You can calculate mass balances in moles, grams and liters for any reaction. The species does not need to exist in the HSC databases.

Fig. 5. Input data for Reaction Equation calculations.

Fig. 6. The equilibrium constant K is equal to the vapor pressure in atm according to Equation (1) if the activity of magnesium is 1. The boiling point of magnesium is about 1100 °C beyond which its vapor pressure exceeds 1 atm.

Fig. 7. Write the reaction equation without stoichiometric coefficients and press Balance Equation.

Fig. 8. Press Calculate to display the results.

Fig. 9. Results for an aqueous ionic reaction.

The data used to calculate the results can be displayed by selecting one substance in the Reaction Equation box, see Fig. 3 and pressing Peep Database. The same procedure can be found in the Results window, see Fig. 4, by pressing the right mouse button. The database content is shown in Fig. 10. Insert is available for inserting a selected formula into the Reaction Equation box. Remove will remove a selected formula from this same box. Note that the selections and editing in Fig. 10 do not have any effect on the HSC databases.

Fig. 10. The database window.

10.4	Reaction Equations Results

The operation of the buttons in Fig. 3 are described in the previous chapter. The meaning of the results can be summarized as the following:

- If the equilibrium constant K is < 1 (or log(K) < 0) the reaction goes to the left.

- If the equilibrium constant K is > 1 (or log(K) > 0) the reaction goes to the right.

- Negative Enthalpy H of the reaction means that the reaction is exothermic, i.e. heat is released, Equation 7 in 8. Introduction.

- Positive Enthalpy H of the reaction means that the reaction is endothermic, i.e. heat is needed, Equation 7 in 8. Introduction.

- Delta Format has no effect on the results of reaction equations.

- In ionic reactions POTENTIAL E yields the electrochemical potential (in Volts) versus the Standard Hydrogen Electrode.

- Equilibrium constant K is calculated using Equation 11 in 8. Introduction.