27. 	Excel Add-In Functions

27.1	Getting started

With HSC 6.0 Add-In Functions it is possible to use the HSC 6.0 database directly under MS Excel 2000 and in that way carry out several thermochemical calculations. In order to use these functions in Excel they must, however, first be activated. Three stages may be needed to activate HSC functions in Excel. The number of stages needed depends on the computer settings and Windows and Excel versions.

Activation Stage 1:

1. 	Open Excel 2000 (NOTE: HSC Add-Ins may not work under earlier Excel versions!)

2. 	Select “Tools, Add-Ins...” from the menu.

3.	Select “Browse…” and locate HSC6.XLL from your HSC6\AddIns folder.

4.	Select “HSC 6.0 Functions” and press “OK”, see Figure 1. It may be necessary to restart Excel in order for the add-in functions to work.

Figure 1: Adding/removing HSC 6.0 Functions under Excel Please do NOT select old HSC 5.1 Functions!.

Activation Stage 2 (Optional):

If the installation is unsuccessful,  the following steps may help:

1. 	Start the VBA Editor by pressing Alt+F11.

(or select Tools, Macro, Visual Basic Editor, ...)

2. 	Select “Tools, References” from the menu.

3. 	Select “Browse…” and locate HSC6.XLL from your HSC directory (for example C:\HSC6\HSC6.XLL). It may be necessary to restart Excel in order for the add-in functions to work. HSC Add-In functions locate in HSC6.DLL, however, Excel calls these functions through HSC6.XLL interface.

4.	See also stage 3 in Chapter 27.2 if needed.

The HSC installation routine automatically takes care of the HSC6.DLL registration.

Activation Stage 3 (Optional):

1. 	During opening, answer No when prompted by Excel to update all linked information (automatic link updates do not work due to some bug in MS Excel).

2. 	Select “Edit, Links” from the menu.

3. 	Choose the path containing HSC6.XLL from the listbox and press “Change Source”.

4. 	Browse to your HSC6\AddIns folder and choose HSC6.XLL.

27.2	Updating XLS-Files which use old HSC 5.1 Add-In Functions

Figure 2: Update XLS Files.xls macro may be used to update old Excel files with HSC 5.1 Add-In functions to HSC 6.0 Add-Ins.

You must update old Excel files with HSC 5.1 Add-In functions. If you find following type formulas from your Excel spreadsheet then you must update:

='C:\HSC5\AddIns_BackUp\HSC5.xla'!H(D11;E11)

HSC 5.1 use old HSC5.XLA interface, HSC 6.0 Add-Ins use better HSC6.XLL interface which makes the use of these files much more easier because the path to the HSC6.XLL files is not saved into the XLS-file cells!

27.3	Brief Description of the Functions

The " AddInSample.xls" sample file offers the fastest way to start using HSC 6 Add-In functions, you may found it from HSC 6 folder, such as:

C:\HSC6\AddIns\AddInSample.xls

The add-in functions are used the same way as functions in general under MS Excel. For example by writing “=H(A1;A2)” the enthalpy for the species in cell A1 and at the temperature in cell A2, is returned. To view all existing functions simply select “Insert, Function” from the main menu and then choose “User Defined” from the left listbox. The right listbox will now give show all available HSC functions and their arguments. A complete description of the functions is given in the following tables 1 and 2.

A useful Excel example file is located in the catalogue “\AddIns” in your HSC installation directory. The example, called “AddInSample.xls”, can be viewed after the add-ins have been made available (described in Chapter 27.1). Figure 3 shows what the example file should look like using MS Excel 2000.

The functions are all collected in the column “Function” and their return values under “Return value”. A red font indicates the input values with a short description of every function shown to the right. This example provides an easy method for testing the functions and also provides practice in learning how to use them.

Table 1: Description of all currently available HSC Species based add-in functions.

Table 2: Description of all currently available HSC Stream based add-in functions.

Figure 3: Example including all available Excel add-in functions (AddInSample.xls).

27.4	Stream Equilibrium Function (Array Functions)

The array functions are created in different way than normal spreadsheet functions. The most important array function of HSC-Sim is StreamEQ which calculates the amounts of species in the equilibrium state at given temperature and pressure.

Fig. 4. Selection for array function.

You may create array function in MS Excel and HSC-Sim using the same procedure:

1.	Create continuos list of phases and species in one column. Important: The phase name string must always start with §-character, Fig. 4.

2.	Specify temperatures of the phases into the next column on the §-phase-row.

3.	Specify the input amounts (moles) of the species into the next column.

4.	Select array function cell range, see Fig. 4.

5.	Type array function: =StreamEQ(A4:A15;B4:B15;C1;C2), Fig. 5.

6.	Keep Crtl + Shift keys down and press Enter, Fig. 6.

7.	The array function is ready if it is within brackets, Fig. 6.

Please note that the array function cell range must be continuous, it is like a solid and fixed block within spreadsheet. Do not brake off this cell range with uncontinuous copy-paste, insert rows, delete rows, etc. operations.

Fig. 5. Type array function.

Fig. 6. Keep Crtl + Shift keys down and press Enter.

27.5	HSC AddIn Functions

Some of the AddIn functions are available on in HSC Chemistry streadsheets but not in MS Excel. Currently the StreamX is such a function:

Function	StreamX

Syntax		StreamX(StremName;ParticlesRange;Variable)

Examples	StreamX(E$6;$E$56:$E$97;$B8)

StreamX(“ROM”;$E$56:$E$97;”SiO2”)

Explanation	Returns the value of the variable, e.g. SiO2 content of the stream calculated according to particles. Note that particle data can be corrected to be in harmony with chemical and mineral composition of the bulk feed.

Applications		StreamX functions is used in Mineral Based Models

Other HSC AddIn functions are StreamX, RecoveryX, ParticleRecX, FractionX, MineralX. These are described in more details in chapter "57. Mineral Based Models".

27.6	More about registering DLL Files

The HSC 6.0 installation routine should take care of all necessary DLL registrations. When selecting/deselecting the HSC 6.0 add-in using Excel, HSC6.DLL is automatically registered/unregistered. However, it is also possible to register/unregister the HSC6.DLL file manually. The program to achieve this is called Regsvr32.exe*) and registration is completed using the following method:

Note: For other HSC6 installation paths than C:\HSC6, simply use your path instead in the instuctions below.

1. 	Select “Start, Run...” from the Windows menu.

2. 	Type “regsvr32 C:\HSC6\HSC6.dll” for registering HSC 6.0 add-in functions.

Alternatively it can be achieved using the following method:

1. 	Double-click the file “HSC6.DLL” in your HSC directory.

2. 	If the file is not registered automatically, which is indicated by an “Open With”-dialog box, choose “Other…” in the dialog window.

3.	Select “Regsvr32.exe” from your Windows System directory. Now double-click the file again and it should register automatically.

When unregistering files follow the same procedure, but add the /u switch before the file name, i.e. “regsvr32 /u C:\HSC6\HSC6.DLL” for unregistering HSC6.dll.

*Description of Regsvr32.exe

To add .ocx and certain .dll files, it will be necessary to run REGSVR32.EXE from the Run option in the Start menu. The following are the commands and switches needed: regsvr32 [/u] [/s] [n] [i [:cmdline]] dll name or ocx name

/u (unregistered server)

/s (silent; display no message boxes)

/c (console output)

/i Call dll or ocx install passing it an optional [cmdline];

when used with /u calls dll or ocx uninstall

/n Do not call dll RegisterServer; this option must be used with /I