# 41.	Sim Flowsheet Example

Fig.1 Sim Flowsheet Example, Run mode view after calculations.

## General

One simple hydrometallurgical simulation example is provided with HSC-Sim. The same basic ideas may also be used in other areas. It may be found from the HSC Chemistry installation folder …\Sim\FlowSheet\Example\Example.fls. It describes the creation of the example in detail with references to the Sim Flowsheet manual. However, there is no need to do everything in the same order as described below.

## Drawing a flowsheet, see Chapter 40.4

- Select the unit icons from the Properties toolbar, Units sheet. If you need to modify the unit icons, see Chapter 40.4.2 and 40.4.3. If you modify or copy the units, remember to change the object type to Unit since the pasted objects are always of the Drawing type, see Chapter 40.3.2 Properties toolbar.

- Draw the streams between the units. One mouse click for the start and to the corners and double click to the end. If the units are Unit-type objects and the stream touches the unit then the source and destination is automatically filled for the stream. For editing the stream afterwards please see Chapter 40.4.1.

- Name the units and streams see Chapter 40.3.2.

- Add the necessary labels. For example, a header label and value labels. Chapter 40.3.14.

- Check the source and destination for the streams. In the example, washing water output of filters is put into the pulping reactor, Figure 2. This is done by filling in the destination row in the Properties toolbar, Process sheet. Chapter 40.3.2.

Fig.2 Changing the destination of the washing water output stream.

- If you need to change the format of the labels or names in the flowsheet you can select all the same kind of labels or names with Edit – Select tool, see Chapter 40.3.1.

## Setting the global measure units, see Chapter 40.5

You find this tool from Excel editor from the bottom-right corner of the window. Click the units with the mouse.

## Setting the variable list, see Chapter 40.5

The sample variable list is used as a base list in this example. You can add the list to the model from the Variable type selection tool by pressing the “Sample variable list” button, Figure 3. You may also fill the variable list manually, the Variable Type Dialog may be used to select the available types for the variables.

Fig.3 Variable type selection window.

You are not limited into the variable types available in the Variable Type dialog, Fig. 3. However, if you create yor own variable types (Type: Other O) you must create also the possible formulas manually. For exampe, "Solid concentration" is not in the sample list and it may be added to the bottom of the list.

- Open any unit in Excel editor and select the input or output sheet.

- Write the type of the variable to the A column (O), name of the variable to the B column (Solid concentration) and unit of the variable to the C column (g/l).

- Make the formula to the D column. In the example the formula is D31/D47*1000, which means that the total solid mass is divided by the total slurry volume and the result is multiplied by 1000 to get the unit to grams solid per liter slurry.

- Copy this formula to the columns E, F and G. These three columns are enough in the example since the maximum number of input or output streams in one unit is three.

- Copy the formula to the other units with the Global cell editor, Chapter 40.15. Open the Cell editor from Excel editor – Tools – Cell editor… Select the first cell where the previously-made formula is and press the Enter button in the Cell editor window. Do the same for all three cells that have the formula. After that the formula is in every unit in input and output sheets. Please see Figure 4.

Fig.4 The Global cell editor.

Solid density is not filled automatically since there is no formula for it. If you need to know a solid volume you have to define the solid density. This can be done with Global cell editor as described above. In the example, a value of 3300 kg/m3 was used.

When modifying the variable list, remember that the columns A to D are automatically synchronized, which means that any changes are automatically copied to all units in the process. Chapter 40.2.3.

Create the formulas for the variables from the Variable type selection – Create formulas all button (or Recreate formulas for all process units button in Global measure units window), Figure 3. This creates the formulas for variables according to the variable type in column A.

## Add reaction equations to the unit, Chapter 40.6.2

The reaction equations can be added with the Distribution wizard. Choose the leaching unit and open the Excel editor ans select "Wizard – Chemical reaction model…" You only have to write the compounds and press the Balance button, which adds the coefficients for the reactions. In the example the reactions are as presented in Figure 5.

Fig.5 Reactions sheet in Distribution wizard.

Remember also to complete the Distributions sheet! It is the same as the Distribution model, Chapter 40.6.1. In the example, 100% of gas phase, go to the Off gas stream and 0% of liquid and solid phase. Fill in the percentage for both streams, i.e. rows 10, 16 and 24 in the example. Press the Save Changes button to accept the changes for both sheets.

In the pulping reactor there are no reactions and only one output stream. The easiest way to make the formulas for the output sheet is with the Distribution wizard. Fill in the rows where there are phase names as described above. Of course you can make the formulas on the output sheet by yourself. In this case, the formula would be the link to the same row in the D column of the input sheets.

## Load and run Excel wizards for the thickener and the filter, Chapter 56

You may create the unit models manually like in Excel, however, the Wizards may offer an eaier way. For the thickener and filter an example wizard is available. You can load it from the Excel editor with selection "Wizard, Load Excel wizards…".

Loading the wizard will only copy it onto the wizard sheet into the unit. Running the wizard will make changes to the unit. In this case when you run the wizards you are asked for the number of different streams in the input or output columns. This is because the streams can be in different columns or have different names. In case of the thickener wizard, if you have a thickener underflow stream in the E column then the underflow is in the 5th column and the overflow in the 6th column.

Fig.6 Wizard sheet in the unit. Wizard-sheet do not carry out calculations, it may be needed to recreate the unit model onto the Model-sheet.

## Setting controls to the process, Chapter 40.12

In the example the solid concentration after the pulping reactor is controlled to 250 g solids/liter slurry. The control is made to the units control sheet. The control sheet is presented in Figure 7. The following rows are filled in.

Table 1. The feed data for controlling the solid concentration after pulping.

| The row name (Cell reference) | Filled value or formula | Notes |
| --- | --- | --- |
| Set point (C7) | 250 | Desired solid concentration, g/l |
| Measured (C8) | =Output!E50 | Measured solid concentration, g/l |
| Tolerance +/- (C9) | 1 | Tolerance for the concentration |
| Value (C9) | =Input!F17 | Water input to the process, t/h |
| X Min Limit (C15) | 0 | Minimum amount for Value cell |
| X Max Limit (C16) | 100 | Maximum amount for Value cell |

Fig.7 The controls sheet of the pulping.

The other control is needed to the leaching to adjust the amount of sulfuric acid fed to the process. In this case we assume that the acid is fully consumed in the leaching.

Table 2. 	The feed data for controlling the sulfuric acid feed for the leaching. In this case the desired amount of acid in the output stream should be equal with the tolerance, then we avoid negative values.

| The row name (Cell reference) | Filled value or formula | Notes |
| --- | --- | --- |
| Set point (C7) | 0.001 | Desired amount of acid in output, t/h |
| Measured (C8) | =Output!F28 | Measured amount of acid in output, t/h |
| Tolerance +/- (C9) | 0.001 | Tolerance for the acid amount |
| Value (C9) | =Input!F28 | Acid input to the process, t/h |
| X Min Limit (C15) | 0 | Minimum amount for Value cell |
| X Max Limit (C16) | 100 | Maximum amount for Value cell |

## Setting feed streams for the process

All feed to the process streams should be set. If the feed to the process is not set the stream is ignored. We have already set some feed streams.

- Washing water for filter is set when we used the wizard for the unit.

- Water feed is set to be dependent on the solid feed to the pulping with the control.

- Sulfuric acid feed is dependent on the acid consumption in the leaching, also set with the control.

The concentrate feed in the example is set to 5 t/h CuS and 5 t/h FeS, Figure 8. Fill in these values to the input sheet of the pulping unit, rows 25 and 26. The correct column is the one that has the right name in row 6. If you have problems to get the correct name to row 6, check that you have the correct destination in the stream information. Properties toolbar – Process sheet – Destination, see Figure 30 in the Chapter 40.4. To refresh the information in the row 6 select Excel editor – Tools – Refresh.

Fig.8 You must specify the external stream amounts and temperatures on the Input-sheet. You should fill only the streams that are coming outside to the process. Internal streams are filled automatically when the process is calculated.

Oxygen feed is the last feed stream in our example. In the example, oxygen in fed to the process 1.1 times the amount required. The formula made to the oxygen feed column (check the row 6 described above) in the input sheet is: =ABS(Model!F12)*1.1

## Saving the process, Chapters 40.2.6 and 40.2.7

Processes should always be saved in their own folder. The change of the process name is not enough since every unit is an Excel file that is saved to the same folder as the flowsheet. The name of these files is same as the name of units. Therefore you have to save also different scenarios to different folders.

## Running the simulation and checking the results, Chapter 40.8 and 40.8.1

When you have finished the model you can run the simulation. To run it you have to go to the Run mode, Run – Start simulation or F5. First, you have to set how many rounds you calculate the process and then press the Calculate button to start the simulation. You should check if the values change during different runs to find out if the process is in balance. Then you have the results and you can select a variable from the Variable selection box. The value of the selected variable is presented in the value labels. Please see Figure 1.

## Useful tools to manage the model

Here are briefly presented a few of the tools provided with HSC-Sim that make the creation of the models easier. These tools are needed in virtually every simulation model.

Global Cell Editor, Chapter 40.15

- If you need the variables that are not present in the Variable type selection, this is the easiest way to copy them to all units.

Variable Type Selection, Chapter 40.5

- It is advised to use ready-made variables when possible.

Distribution Wizard, Chapter 40.6.2 and Excel wizards, Chapter 56

- With wizards you can easily make a new unit.

Element Balances, Chapter 40.10

- This tool is useful for checking if there are any balance errors in the process.