55.	Map Basics

Fig. 1. World Map with Latitude-Longitude coordinates.

Usually the normal HSC Map user does not need to know the map coordinate system details which are summarized in this chapter. However, if you  add your own maps to the HSC Map module “C:\HSC5\Map\Maps” folder, it may be useful to read this chapter first.

The traditional latitude-longitude map coordinate system is shown in Fig. 1. The equator divides the earth into the northern and southern hemispheres. The N and S in the Y-axis scale stand for North and South. The X-axis zero point has been fixed at Greenwich, London. This divides the Earth into western and eastern hemispheres. The W and E stand for West and East in the X-scale.

The location data may be specified using various syntaxes. Traditionally, the Y-axis coordinate is given first and then the X-axis. For example, the location of the Outokumpu Research Oy main entrance (red point in Fig. 1.) may be specified in the following way:

Y-axis	X-axis	Syntax	Datum	Comment

Latitude	Longitude

N 61° 27.727'	E 21° 52.068'	dd°mm.mmm’	WGS-84	Default in GPS

61° 27.727' N	21° 52.068' E	dd°mm.mmm’	WGS-84	Older syntax

N 61° 27' 43.6"	E 21° 52' 4.1"	dd°mm’ss.s”	WGS-84	Seconds also

61.46198333	21.87126667	dd.ddddddd	UTM	Decimal format

N 61° 27.719'	E 21° 52.276'	dd°mm.mmm’	UTM	National

N 61° 27' 43.2"	E 21° 52' 16.5"	dd°mm’ss.s”	UTM	National

6817.447 km	1546.457 km		UTM	National

HSC Map supports all these formats and many others. Digits: d = degree, m = minute, s = second.

55.1	Map Datums

The map datum specifies the Earth’s shape, one fixed point and the scale. Due to historical reasons, countries often use their own national datums in their maps.

The Earth’s shape is far from a perfect sphere. Its shape is fixed by specifying the ellipsoid, which may be used for trigonometrical location calculations, Fig. 2. The Ellipsoid is a smooth mathematical surface that best fits the shape of the earth surface. The required parameters are:

a	=	Earth radius at equator = Semi Major Axis = 6378.137 km (WGS-84)

b	=	Earth radius at poles = Semi Minor Axis = 6356.752 km (WGS-84)

f	=	Flattening = (a-b)/a = 0.0033528106647 (WGS-84)

e	=	Eccentricity = 2 * f – f ^ 2

Fig. 2. Earth shape modelled with a geometrical ellipsoid.

The current worldwide ellipsoid standard is called the WGS-84 (World Geodetic System 1984). This datum is used commonly in GPS navigators (GPS = Global Positioning System).

National datums may be converted to this format using the national ellipsoid data with DX, DY and DZ parameters which describe the national datum shift to the WGS-84. Ie. the national datum fixed point shift to the WGS-84. The basic datum-point of Finland is the Observatory of Helsinki. These shift values specify the origo shift in the Cartesian based x-y-z-coordinate system in the middle of the earth.

The HSC Map Datum Database contains several national datums, i.e. specification of the ellipsoid and shift values to the WGS-84 datum. You may see this data by selecting “Tools, Datums,…”. You may also add your own data to this file, for example, using MS Excel. However, do not add new columns to this file. The path of this file is usually: “C:\HSC5\Map\MapDatum.xls”.

The difference between the national datum location points is usually less than 200 – 300 meters compared to WGS-84. For instance, this difference cannot be seen on the large-scale maps like Fig. 1. In Finland the shift is roughly 200 m. The HSC Datum Converter may be used to estimate the effect of the Datum shift on the location coordinates. Select “Tools, Translator 3…” from the main menu, Fig. 3.

Fig. 3. Map Datum Converter converts Datum 1 location data (decimal format) to Datum 2 location data. Shift values DX, DY and DZ describe the difference between Datum 1 and 2 fixed points in x-y-z-coordinate system, which origo is in the middle of the Earth. X-axis points to Greenwich, Y-axis to Equator 90° East from Greenwich and Z-axis points to North Pole.

55.2	UTM Coordinates (Kilometer Coordinates)

Fig. 4. Default UTM zones of the world.

National maps are typically based on kilometer coordinates, because the distances and areas are easy to calculate on these maps. These km-based coordinates are, however, valid only within a limited latitude-longitude range, because the Earth is spherical and a ball-shaped surface is impossible to convert to two-dimensional form without a large distortion, Fig. 4. However, for a limited surface range it is possible to carry out the conversion with reasonably low distortion and error.

The UTM (Universal Transverse Mercator) projection is the most common method to convert the limited area of the spherical latitude-longitude map in a flat two-dimensional form, Fig. 5. Usually the width of the UTM zones is 6°.

Fig. 5. The default width of the UTM zone is 6°.

The UTM zone Y-coordinates describe the distance from the equator. The X-coordinate describes the distance from the UTM zone central meridian (i.e. longitude line from North to South Pole). Commonly, in order to get nice looking number values the False Easting and False Northing parameters are used.

Fig. 6. Currently used UTM (Gauss-Krüger) zones in Finland. Central meridians are 21°E, 24°E, 27°E and 30°E. These maps are also called KKJ-maps.

Finland uses slightly modified UTM projection type (Gauss-Krüger), the zone width is only 3° and the Scaling Factor is 0.9996 instead of 1.0. The basic datum-point of Finland is at the yard of Observatory of Helsinki.

Small countries often use only one UTM zone, however, large ones need more. For example, Finland uses 6 zones of which the central meridians are: 18, 21, 24, 27, 30 and 33°. The most common ones are 21, 24, 27 and 30°, Fig. 5. At the west coast the following UTM parameters are used:

-	Central Meridian	21°

-	False Easting		-1500 km

-	False Northing	0 km

These values specify the coordinate scale which is used in Finnish UTM (Gauss-Krüger) maps.

Fig. 6. Latitude, Longitude – UTM Grid Converter.

You may use the HSC Map Latitude, Longitude – UTM Grid Converter to test the effect of the UTM datum on the coordinate values, Fig. 6. The UTM converter may be opened from the HSC Map main menu “Tools, Translator 2…” selection.

You may see some national UTM parameters by selecting HSC Map main menu “Tools, Datums,…”. You may also add your own data to this file, for example, using MS Excel. However, do not add new columns to this file. The path of this file is usually: “C:\HSC5\Map\ MapDatum.xls”.

IMPORTANT:

- If you add and calibrate new maps to HSC Map please remember to select the valid map Datum first. If you select the wrong one the coordinates will be in error.

- HSC Map version 1.0 supports only UTM and Latitude-Longitude maps with the same Y- and X-scale. These maps are often called Equirectangular Mercator Projection, Cylindrical Equidistant Projection or Plate Carr maps.

- HSC Map supports origo shift and rotation in all maps.

55.3	HSC Map Datum Database

Fig. 7. HSC Map Datum Database.

You may open HSC Map Datum Database using main menu selection “Tools, Datums…” or you may use Excel to open the file:

C:\HSC5\Map\ MapDatum.xls

This file do not contain all possible datums (some 12000 may be found) but you may easily add the missing datatum by inserting a new row to the datum table. However, do not insert or delete colums, because then HSC Map reads the datum record data from the wrong data fields.

55.4	Location Data Error Sources

The GPS navigator gives usually the error in meters, it depends on the local conditions and the number of visible satellites. Usually the error is a few meters.

The national datum shift compared to WGS-84 is usually less than 300 meters, in Finland this shift is some 200 meters. The Datum conversion error using DX, DY and DZ parameters is some meters because it does not take into account the Scaling Factor and rotation shift of the x-y-z-coordinates.

HSC Map saves the Stock Place location data using national datum because the DX, DY and DZ parameters may vary.

The Latitude-Longitude and UTM conversion error is usually small within the given UTM zone, however, outside this zone the error increases a lot.

The rounding-up error depends on the location value syntax. One degree is roughly 111.3 km and one minute is 1.8 km and one second is 30.9 m. On this basis some examples of the the syntax errors may be given as:

Syntax	Rounding-up Error (meters)	Comment

N dd.mm.ss	15.46 m	Seconds

N dd.mm.mmm	0.928 m	Decimal minutes

N dd.ddddd	0.556 m	Decimal degrees

N dd.dddddd	0.056 m	Decimal degrees

The North and South Pole and continents move slowly, the dimensions of the earth change, the WGS-84 ellipsoid specifies the earth shape approximately. So nobody knows which are the absolutely correct place coordinates.