# 57. Mineral Based Models

## General

HSC-Sim has a special set-up and approach for processes where minerals are treated. In the processes including crushing, grinding, flotation, gravity separation or screening this approach should be used. In mineral-based models particles are the treated which have at least the following properties:

- size (diameter)

- mineral composition as wt.%

In addition, they may have additional parameters like mineral volume%, mineral area% (on surface of a particle), whiteness and hardness. Globally, minerals have a certain chemical composition and specific gravity and therefore HSC calculates these properties for each particle and also for each stream.

Particles can be generated through mineralogical information. There are two levels of information required (Figure 1):

Figure 1. Global and set-up for each input stream.

- Global

Global mineralogical information does not change from stream to another. Global information consists of:

  - A list of minerals and their properties including:

    - Name

    - Chemical formula

    - Specific gravity

    - Chemical composition

    - Number of behavioral types and their names (e.g. n=2, fast floating, slow floating)

  - Size class information including

    - Number of size classes

    - Lower and upper boundary of each class and average

    - Name of the size class

- Each input stream

Each input stream defines the mineralogical composition of the stream by size and possibly by behavioral types. An input stream data consists of:

    - Mineral data

    - Total input of the stream (t/h)

    - Weight percentage of each size class (summing to 100%)

    - Weight percentage of each mineral in each size class (summing to 100%) and in the bulk (calculated)

    - Distribution of minerals between behavioral types

    - Chemical composition of fractions and the bulk (calculated)

    - Particle data (optional but if missing is generated)

    - In the model particles are used, i.e. if this data is missing, then particles are generated according to (a) and (b) or (a) and (c). If given (e.g. from the liberation analysis) the weight percentage of a mineral is corrected according to data given in (a), i.e. the mineral weight percentages are taken from the data (a) whereas mineral association and mode of occurence of data is taken from (d). This is important since liberation analysis is often biased in terms of modal composition. Also, it enables to give more information on trace minerals, e.g. gold, than the simple liberation analysis would provide.

In units particles (Figure 2):

- Can break down (crusher, mill),

- Can distribute between the output stream either perfectly or with certain ratios (probabilities) which can be dependent on:

  - Size

  -  specific gravity

  -  mineral composition

- Can travel through the unit unchanged but mixed with other streams.

Figure 2. Behavior of particles in a unit.

## Setting-up the mineral-based models

To set-up a circuit based on particles, consisting of minerals with a certain chemical composition:

- Draw the process with all units and streams (Figure 3)

- Select Data – Mineral Setup… from the menu (Figure 4)

- In the Mineral Setup window follow the Step-by-Step setup to define (Figure 5, see a more detailed description of step 3 operations on following pages):

1. Minerals

2. Size classes

3. Streams

4. Variables

after these stages

5. Save and

6. Close

- Do the appropriate set-up for each unit (using Excel wizards)

Figure 3. Draw the process.

Figure 4. Select Data – Mineral Setup…

In the mineral set-up window you can edit the mineral setup either using step-by-step approach (select Step by Step option) or edit the properties independently (select Editor option; Figure 5). The mineral setup has been designed so that the first time the setup is to be done step-by-step whereas minor changes can be done in the editor mode.

Figure 5. Mineral Setup window.

The step-by-step approach is described first (Figure 5).

- The first step   is to define the minerals. To do that Press button “1. Minerals…” If the button is not visible, select the Step-by-Step option. The HSC-Sim Wizard Selection window will appear and certain built-in mineral definition will be available, e.g. “Cu ore (porphyry)” and “Au ore” (Figure 6). Here HSC-Sim reads the Excel file MineralogyWizard at \Sim\Wizards\System –folder. Each selection has its own sheet. To edit, add or remove selection, open the file with Excel and make the appropriate changes. When adding a sheet it may be easier to copy the content from the existing one and then edit this because the structure has to similar to existing sheets (e.g. the mineral number on row 6, C is the first column etc.).

- Complete the Global Mineral set-up by adding them, removing species (columns), editing their chemical composition, their specific gravity and number of behavioral types (Figure 8). In this context, behavioral types mean that if you want to specify a fixed ratio for example fast-floating, non-floating and non-floating fractions. The number of behavioral types can be between 0 and 3. Zero means that there will be one anyway, but the set-up is slightly different if you specify one. The number of behavioral types should be same for all the minerals although it is individually requested.

Figure 6. Select appropriate mineralogy.

Figure 7. Files for mineralogy (MineralogyWizard.xls) and size class (SizeClassWizard.xls) setup are located at Sim\Wizards\System folder.

Figure 8. Step-by-Step mineral set-up. the first step “1. Minerals” completed

- To define size classes, press the “2. Size classes…” button. The wizard window becomes visible. Make the selection most suitable for you. The size class wizard is an Excel file, SizeClassWizard.xls, located at \HS6\Sim\Wizards\System –folder. Each selection is a sheet. You may edit the content of each worksheet, delete sheets or add them using Microsoft Excel. Close the file in Microsoft Excel before running the wizard, because Excel reserves the file totally for its own purposes.

- Edit the size fractions in the Global Size Set-up sheet (Figure 10). Change the unit, upper and lower value, average size and label. You may also delete or add fractions.

- The next step is to define the mineral composition of the input streams. Press the button “3. Streams…”. Fill-in the data of the input stream(s) (Figure 11). Give the total input, wt% of size fraction, mineral composition of each size fraction and fraction of each behavioral type if applicable. To calculate the chemical composition of each fraction and the bulk select Tools- Calculate Chemical Composition.

- The next step is to set the variables to each unit. Do that by pressing “4. Variables” (Figure 12). The operation will take a while. It will create particles (if not defined), and set-up each unit.

- To save the information press “5. Save” and to complete the set-up press “6. Ready”.

Figure 9. Wizard for defining size classes.

Figure 10. Global Size set-up sheet (Size –tab).

Figure 11. Fill-in the data of the input stream(s). Give total input, wt% of size fraction, mineral composition of each size fraction and fraction of each behavioral type if applicable. To calculate the chemical composition of each fraction and the bulk select Tools- Calculate Chemical Composition.

Figure 12. Step 4, press 4. Variables… button.

## Coding in the column A of the units

In the model, the data is specified in the first column (column A). The syntax of the code is:

Particles are referred as

M.P.1 = particle number one (t/h)

The following items are calculated from the particle amount information (M.P.1 – M.P.n) each time the model is run. They are properties of each stream.

The codes are:

| M. | M. | Size. | Mineral. | Type. | Item |
| --- | --- | --- | --- | --- | --- |
| Options |  |  |  |  |  |
| M=mineral | M=mineral E=element L=liquid S=slurry P=Particle | 1,2,3... = Integer giving the number of the size class 0 = all minerals (i.e. bulk) | 1,2,3... = Integer giving the number of the mineral (M) or element (E) class 0 = all minerals (i.e. bulk) | 1,2,3... = Integer giving the number of the type 0 = all types (i.e. bulk) | a=amount t=t/h w=wt.% f=fraction (w/w) r=recovery s=SG v=volume |
|  |  | For Particle is the input stream index | Number of particle in input stream |  |  |
| Examples |  |  |  |  |  |
| M. | M. | 0. | 0. | 0. | a |
| M.M.0.0.0.a Total solids mass flow of the stream |  |  |  |  |  |
| M. | M. | 1. | 0. | 0. | w |
| M.M.1.0.0.w Weight percentage of the first size class in a stream |  |  |  |  |  |
| M. | M. | 0. | 2. | 0. | w |
| M.M.0.2.0.wt Weight percentage of mineral 2 in a stream |  |  |  |  |  |
| M. | E. | 1. | 1. | -. | w |
| M.E.1.1.-.wt Weight percentage of element 1 in a stream |  |  |  |  |  |
| M. | E. | 0. | 3. | -. | r |
| M.E.0.3.-.r Recovery of element 3 in the stream (against total input) |  |  |  |  |  |

## Functions available in Excel models for mineral based models

Preparation of the variable list with the Mineralogical Set Up window will bring the number of variables into the units. Functions can be entered also by the user and will not function in Excel.

### Stream functions

| Function | StreamX |
| --- | --- |
| Syntax | StreamX(StremName;ParticlesRange;Variable) |
| Examples | StreamX(E$6;$E$56:$E$97;$B8) StreamX(“ROM”;$E$56:$E$97;”SiO2”) |
| Explanation | Returns the value of the variable, e.g. SiO2 content of the stream calculated according to particles. Note that particle data can be corrected to be in harmony with chemical and mineral composition of the bulk feed (see xx). |

| Function | RecoveryX |
| --- | --- |
| Syntax | RecoveryX(Variable;StreamName;FeedName) |
| Examples | RecoveryX($B21;E$6;0) RecoveryX(“Fe”;”Zn Conc”;”ROM”) |
| Explanation | Returns the recovery of variable in the stream vs. the feed (or another stream). The RecoveryX add-in function cannot be used in the Target cell, because it is recalculated only after all calculation Rounds have been finished. |

### Particle functions

| Function | ParticleRecX |
| --- | --- |
| Syntax | ParticleRecX(ParticleID;ModelReference;TableOfVariables) |
| Examples | ParticleRecX(A56;1;Model!B22:F34) |
| Explanation | Returns the recovery value of mineral according to TableOfVariables. Model reference: 1= wt average based on minerals, 0= returns the value according to particle size. |

### Fraction functions

| Function | FractionX |
| --- | --- |
| Syntax | FractionX(FractionNumber;FractionVariable) |
| Examples | =FractionX(A2;B2) =FractionX(1;”Label”) |
| Explanation | Returns the value of the variable (Label, Lower, Upper, Average Size) of the Fraction in question (FractionNumber). |

### Mineral functions

| Function | MineralX |
| --- | --- |
| Syntax | MineralX(MineralNumber;Variable) |
| Examples | =MineralX(A2;B2) =MineralX(1;”Cu”) |
| Explanation | Returns the value of the variable (chemical elements, S.G., Code, Mineral) of the mineral in question (FractionNumber). |