BioClock User’s Guide

# ZKSoftware Inc.

# Table of Contents

1. Introduction……………………………………………………………3

1.1 Basic Concepts…………………………………………………….5

1.1.1 User Enrollment……………………………………………….5

1.1.2 User Verification……………………………………………….6

1.1.3 Match Threshold Levels……………………………………….6

1.1.4 User ID Numbers………………………………………………7

1.1.5 Authority (status) Levels………………………………………7

1.1.6 Start-up window………………………………………………8

1.2 How to place the finger…………………………………………..8

2. Enrollment and Verification Procedures…………………………...10

2.1 Enrolling User…………………………………………………..11

2.2 Testing an Enrollment………………………………………….18

2.3 Enrolling a fingerprint for backup………………….…………..19

2.4 Verifying Your Identity………………………………………...19

2.4.1 Fingerprint Verification…………………………………….20

2.4.2 Password Verification……………………………………….21

2.4.3 ID & Fingerprint……………………………………………22

2.5 Hints for Successful Enrollments……………………………...23

3. System Options…………………………………………………….25

3.1 System Options………………………………………………….25

3.1.1 Date Time…….………………………………………………26

3.1.2 Language…………………………………………………….27

3.1.3 Lock Controller……………………………………………..27

3.1.4 Advanced Options……………...………………………………27

3.2 Power Manager…………………………………...………………..28

3.3 Communication Option……………………………………………..30

3.4 Log Options…………….……………………………………………31

3.5 Auto Test…………………………………………………….………..31

4. System Information…………………………………………………….33

5. Maintenance…………………………………………………………34

1. Introduction

This chapter shows the BioClock Fingerprint T&A basic concepts and how to place the finger.

## Figure 1~1 showing

BioClock I                                         BioClock II

BioClockIII                                             F4

### Figure 1~2 keypad

Keypad Definition

Confirm the current present.

Access to manage.

Escape or cancel.

Scroll up.

Scroll down.

Power button.

ˉˉ：Numerical key.

1.1 Basic Concepts

This section contains definitions and descriptions of BioClock Fingerprint T&A Concepts, including:

·User Enrollment

·User Verification

·Match Threshold Levels

·User ID Numbers

·Authority (Status) Levels

The two most important BioClock Fingerprint T&A Functions are user enrollment and user verification.

1.1.1 User Enrollment

Enrollment is the process of create an ID number and scanning a user’s finger to create a template three times. This template is associated with a user ID number and stored.

During user verification, the stored template is compared to the current fingerprint scan to confirm the user’s identity.

Enrollment is performed at designated BioClock units. The user can verify on the time recorder that enroll his fingerprint. The enrollment process takes approximately 2 seconds.

Up to ten fingers can be enrolled for the same ID number, so the user may have more verifications way.

Ideally, everyone finger on each hand should be enrolled so that if the user injures the enrolled finger had, an alternate finger template is available. While recording the finger type is optional, it is recommended such as left or right index, so that at verification the user is prompted as to which finger to use.

1.1.2 User Verification

Verification occurs when a user either enters an ID number or places a finger in the fingerprint sensor, then input password for comparison with the stored template.

1.1.3 Match Threshold Levels

The Mach Threshold is a number that represents the degree to which a verification is examined to ensure that the person is the person claimed.

The Match Threshold Levels establishes a balance between False Acceptance Rate (FAR) and False Rejection Rate (FRR). FAR measures how often a non-authorized user is falsely recognized and granted access to the system. FRR measures how often an enrolled and authorized user, who should be granted access to the system, is denied on the basis that the system did not recognize him.

You can set match threshold levels on a per-use basis. For the fingerprint verification is difficult, you can adopt ID & Fingerprint verification (match one to one).

Raising the threshold increases security, while lowering it increases throughout. The correct balance is essential.

For users with worn or damaged fingers, match threshold levels can (and should) be reduced.

Different sites have different requirements.

Table 1—1 Suggested Match Threshold Settings

FRR FAR One-to-many One-to-one

High Low 45 25

Middle Middle 35 15

Low High 25 10

1.1.4 User ID Numbers

Before beginning fingerprint enrollment, a user is assigned User ID Number. This ID number is used to call up the fingerprint template each time that verification is requested.

ID numbers are normally called up via the keypad, but they can be called up through other means such as RF Card.

1.1.5 Authority (status) Levels

BioClock Fingerprint T&A has four authority or status levels:

·Users are people whose identity must be verified, such to gain access to a facility or to have their attendance recorded.

·Enrollers are Users who are authorized to enroll new users or delete on to the system.

·Managers can do other operations, except set advanced option and enroll manager’s authority.

·Supervisors are Users who access to all functions and change all setup in the system.

Note: Without Manager and Supervisor status in the system, the Enroller will enroll them. And if there isn’t a Supervisor in the system, the Manager will enroll him.

1.1.6 Start-up window

Click the power button and the screen showed on, a display of a window was called the Start-up window. The following appears:

Welcome

04-01-02       FRI

1.2 How to place the finger

- The correct way is:

###### Place a finger flatwise on the sensor surface

###### Place it to the center of the sensor surface

- The wrong way is:

askew

part

Upright

Uncentering

Note: Please adopt the correct way to place the finger, this company assumes no responsibility for errors from the use of the BioClock. This company reserves the right to revise document and to make changes from time to time in the contents here of without obligation to notify any person of such revision or changes.

2. Enrollment and Verification Procedures

This chapter describes how to enroll and verify users on the Fingerprint T&A verification system.

The following topics are included:

·Enrolling User

·Testing an Enrollment

·Enrolling a User with the Display Finger Option On

·Verifying Your Identity

·Hints for Successful Enrollments

Note: You must have Enroller, Manager, or Supervisor status to enroll users. For information on status levels, see “Authority (Status) Levels” on 1.1.5.

2.1 Enrolling Users

If this is the first enrollment in a new or empty system, everyone will become an enroller.

If there is a manager in the system, you will have Manager status to enroll a new user.

There are three ways of enrollment, fingerprint enrollment, password enrollment, fingerprint & password are suitable for three differences crowds. Fingerprint enrollment suited the most people who have better quality fingerprints; Fingerprint & Password suited few people who enrolling successful, but verification was difficult; Password enrollment suited 1% people who verification unsuccessful.

To start the enrollment process, identify yourself firstly—press [Menu], enter your ID number or fingerprint, and then verify your identity.

Note: If this is the first enrollment in a new or empty system, you will not be prompted for verification.

The following message appears on the display:

Menu

► User Manage

Options

Sys Info

Press [OK], access the User Manager, the following message display:

User Manage

►Enroll User

Enroll Admin

Delete

Press [OK], access the User Enrollment, the following message display:

Enroll User

►Enroll FP

Enroll Pwd

FP & Pwd

- Fingerprint Enrollment

(1) Access the Enroll Fingerprint to press [OK], the following display:

New Enroll

Enroll No 00001

ESC          OK

(2) Press [OK], the following appears:

New Enroll

Enroll No 00008

ESC          OK

(3) Input the enrolling number (the range is 1 to 65534), press [OK], the following appears:

New Enroll

Place Finger…

ESC

(4) If the test is successful three times in a row, the following appears:

New Enroll

00008-1

ESC      OK [Save]

Note: 07711-1

The last number 1 means the first fingerprint.

Press [OK], the previous message continues to be displayed while the template is created.

If your identity cannot be verified, you are prompted to try again. And you must restart the verification procedure (step 3).

2. Password Enrollment

(1) Access the password enrollment, press [OK], the following appears:

Enroll Pwd

New Enroll?

ESC           OK

(2) Press [OK], the following appears:

New Enroll

Enroll No 00008

ESC         OK

(3) Input the enroll number (the range is 1 to 65534), press [OK], the following appears:

New Enroll

Input Pwd *****

ESC        OK

Note: The password range is 1 to 5.

(4) Input the password, press [OK], the following appears:

New Enrollment

Input Pwd: *****

Pwd Affirm: *****

ESC          OK

(5) Input the password again, press [OK], the following appears:

New Enroll

00008-P

ESC     OK [Save]

Note: 00008-P

The last letter P means password.

Press [OK], the previous message continues to be displayed while the template is created.

3.Fingerprint & Password

- Access the Fingerprint& Password, press [OK], the following appears:

FP & Pwd

New Enroll？

ESC          OK

(2) Press [OK], the following appears:

New Enroll

Enroll No 00008

ESC          OK

(3) Input the password (range is 1 to 65534), press [OK], the following appears:

New Enroll

Please finger…

ESC

(4) If the test is successful three times in a row, the following appears:

New Enroll

Input Pwd: *****

ESC           OK

(5) Input your password, the following appears:

New Enroll

Input Pwd: *****

Pwd Affirm: *****

ESC          OK

(6) Input the password again, press [OK], the following appears:

Note: 00008—1P

New Enroll

00008-1P

ESC      OK [Save]

The second from the last 1 means one fingerprint

The last letter P means password.

Press [OK], the previous message continues to be displayed while the template is created.

2.2 Testing an Enrollment

Ask the users to place their finger to perform a test verification. If the test was successful, you will adopt Fingerprint enrollment. If the poor quality of fingerprint, you will be recommended to use Fingerprint & Password.

2.3 Enrolling a User with the Display Finger Option On

Press [ESC] to the Display Dinger Option On, the following appears:

New Enroll

Enroll  No 00008

Esc          Ok

Display

Enroll No 00008

Esc             Ok

ESC

Note: Where system memory permits, it is advisable to have at least two fingers enrolled for a long-term user.

2.4 Verifying Your Identity

The workers used Fingerprint T&A to verify their identity. The system defaults to three ways of verification: Fingerprint verification, Password verification, ID number & Fingerprint. The following appears:

2.4.1 Fingerprint Verification

Place the finger on the sensor surface, the following appears:

FP Verify

Please Move Finger

It continues for about 0.5 seconds. If the test is successful, it will say “Thank you”, the following appears:

FP Verify

Enroll No 00008

Verified!

If your identity cannot be verified, you are prompted to try again, the following appears:

.

FP Verify

Place Finger Agn！

It continues for about 0.5 seconds, return to the start-up window.

2.4.2 Password Verification

To start the enrollment process, enter your ID number, the following appears:

1:1 FP Verify

Enroll No 8

ESC      OK [Pwd]

Press [OK], the following appears:

Pwd Affirm

Enroll No 8

Input Pwd: *****

Input correct password, press [OK], the following appears:

Pwd Affirm

Enroll No 00008

Verified!

If the password cannot be verified, the following appears:

Pwd Affirm

No   8

Error pwd !

2.4.3 ID number & Fingerprint

To start the enrollment process, enter your ID number, the following appears:

1:1 FP Verify

Enroll No 8

ESC      OK [Pwd]

If your identity is verified, the following appears:

1：1 FP Verify

Enroll No 00008

Verified!

2.5 Hints for Successful Enrollments

The fingerprint is of good quality, the verification speed will fast; Otherwise, it will verify slowly or occur to FRR.

In order to improve the fingerprint verification quality, the following hint appears:

Table 2—1 Enrolled unsuccessful or the fingerprint of poor quality

| Dry or dirty fingerprints | Solve the dry problem that rub the dry finger and palm. If the fingerprint is dry, you will adopt the way of wetting up the finger, such as 哈气. |
| --- | --- |
| Have no enough to bring pressure | The user should place a finger firmly and squarely on the sensor surface. |
| How to select the finger? | Recommend to left or right index or middle finger. Use fingerprints of good quality, without worn or hurt. The user usually selects the forefinger, if it was of a poor quality, you will be recommended to middle finger or ring finger. If the user’s finger is less, you will select the thumb. |
| How to place the finger? | Place your finger firmly and touch the sensor surface must be over 2/3. The fingerprint does not touch upright on the sensor surface. Do not touch the finger too fast; Do not move the finger on the sensor surface. |
| The influence of the fingerprints change | For users with worn or damaged fingers, the identification can be influenced. If the finger quality was poor, you will select password verification. |
| Others | However, few people’s fingerprint quality is too poor to verify the fingerprint in gear. Please use ID & fingerprint verification, and the Match Threshold Levels can be reduced or use password verification. |

3. System Options

Press the [Menu], then verify your identity. The following appears:

Menu

User Manage

Options

Sys Info

Access Options, press [OK], the following appears:

Options          ▼

System Opt

Power Mng

Comm Opt

The following topics included: System Options, Power Manage, Communication Options, Log Options and Auto Test.

3.1 System Options

Access System Opt, the following appears:

System Opt      ▼

Date Time

Language      ENG

Lock          No

The setup facility covers four areas: System Options, Language, Lock and Advanced Options.

3.1.1 Date Time

Access Date Time, the following appears:

Date Time

2004-01-02

08:24:35

ESC           OK

To change the date, press and , then input the correct date and time, press [OK]

3.1.2 Changing the Default Language

Access Language, press [OK], it defaults to SCH. Press  to select another language. Fingerprint T&A comes with three built-in languages, simplified Chinese, traditional Chinese, and English for all displays. Select the language that you will use, press [OK], then Press [ESC] to return the Options. The system will ask for confirmation of the displayed language. Press [OK] to accept the language displayed.

3.1.3 Lock Controller

It defaults to close the lock signal output in the system. If you use access control feature, you will change [No] to [Yes], press [OK] to confirm, and then press [ESC] to return the Options. The system will ask for confirmation of the displayed controller, press [OK] to confirm.

3.1.4 Advanced Options

Access Advanced Option, press [OK], the following appears:

Adv Option       ▼

Restore Deflt

Del Logs

Clear all Data

Press and  to scroll up or down the screen to select the option.

Restore Default: restore all setup information to default.

Clear all Data: delete all enrolling fingerprints and logs.

Delete Logs: delete all logs of flash disk.

Clear Managers’ rights: change the managers’ rights into ordinary users’ rights.

Show Score: whether show the quality value of fingerprint in the screen or not;

Match Threshold Levels: For helping in selecting threshold level settings, see Table 1.1.3.

Sound: whether use to phonic hint or not.

3.2 Power Manager

Access Power Manage, the following appears:

Power Mng     ▼

Shutdown     No

Power On     No

Sleep         No

Power Mng       ▲

Sleep          No

Idle           Off

Idle Min        0

This product uses smart manage system, supports the time switch and resting features, it can satisfy the user’s different demands.

Shutdown: auto shut-off in the timed.

Power on: auto open-up in the timed.

Sleep: automatic resting in the timed, press any key to enter working state.

Idle and Idle Minute, they are related each other, while the idle minute is zero, the idle is closed; while the idle minute is not zero (unit was minute), for example, it is one minute, the user does not do anything in one minute, the system will enter the idle state.

Select Shutdown, press [OK], the following appears:

Shutdown

Setup On time Fun?

ESC          OK

Press [OK], then enter Shutdown, press [ESC] to cancel.

3.3 Communication Options

Access Comm Opt, the following message appears:

Comm Opt           ▼

Dev Num           1  Baud Rate      1115200

IP address

Comm Opt     　▲

Ethernet         No

RS232        Yes

RS485        Yes

BioClock Fingerprint T&A support RS232, RS485, TCP/IP, whether you use Single-Board Computer (SBC) or Networking, BioClock will satisfy the user’s demands.

Device Number: machine ID, the range is 1 to 255.

Baud Rate: there are three options, 9600, 38400, 115200; The speed of communication is fast, recommend to use RS232; The speed of low communication is stable, recommend to use RS485.

IP address: it defaults to IP address is 192.168.1.201.

Ethernet: whether use TCP/IP protocol or not.

RS232: whether use RS232 or not.

RS485: whether use RS485 or not.

3.4 Log Options

Access Log Opt, the following message appears:

Log Opt

Alm AttLog      1

ReCheck Min    0

Alarm Attendance Log: when the remainder log capacity reached set numerical value, it will automatically sound a warning that logs were fulled.

Recheck Minute: Set within compass (Unit: minute). Someone’s enrollment has enrolled, then, the log twice was not displayed in the system.

3.5 Auto Test

Access auto test, the following message appears:

Auto Test        ▼

Run All Test

Flash Test

LCD Test

Auto Test        ▲

FP Reader

Key Test

RTC Test

In the option, you can run system device test. When the device broken down, it can analyse the cause of the device’s fault and the devices were quickly and easily maintained.

It tests the Memory, LCD, Sound, Fingerprint sensor, keypad and clock. In the course of test, you should guarantee the stability of the power. Otherwise, the system’s hardware was probably damaged, especially it runs the memory test.

4. System Information

Access [Menu] to Sys Info, press [OK], the following appears:

Sys Info         ▼

User Cnt     206

FP Cnt       173

Att Log      8046

Sys Info         ◆

Admin Cnt      0

Pwd User      37

Res. Space Inf

Sys Info         ▲

Pwd User      37

Res. Space Inf

Dev Info

It displays user numbers, password numbers, fingerprint numbers, manager and log numbers in the screen. You can see some information such as the remainder log capacity in the memory, the device capacity, the defaults date, machine number and OEM’s information are in the system.

5. Trouble shooting

1. Cleaning

From time to time, the optical platen, the keypad and display window require cleaning. Since working environments differ, it is not possible to define when cleaning should be performed. Following is guide:

| ##### Item | ##### Cleaning Frequency |
| --- | --- |
| Keypad and display window | Cleaning when visibly dirty and hard to read. See cleaning the keypad and display below. |
| Optical platen | Do not over clean. The platen is designed to work under greasy or dirty conditions. |
|  | However, do clean if the platen is obstructed or people are reporting deteriorating performance. See cleaning the Optical Platen below. |

Table 5-1 Cleaning Schedule

2. Cleaning the Keypad and Display

To clean the keypad and display, use the same cleaning products as for the platen, and wipe dry.

3. Cleaning the Optical Platen

Clean the optical platen as follows:

(1) If dusty or gritty, first blow on the platen to clean off any loose particles.

- Spray on a light coating of a window cleaning product or other similar neutral detergent.

Warning: Do not use any other cleaner or the platen may become damaged.

(3) Using a non-abrasive and lintine, pat dry. Be careful not to scratch the platen. If lint particles adhere to the platen surface, below them off when the platen is dry.